Manual For Online Map Submission,

Automated Building Plan Scrutiny

&

Approval System

- PROMINENT PROCESS OF THE SOFTWARE

Automation of Online Map Submission, Automated Building Plan Scrutiny and Approval System is the main part of the system which is used by Architects / Consultants as well as Officers.

  - PRECHECK ENGINE

Checks the Geometry compliance of the Submitted drawing by verifying the color codes, layers, mandatory items, text, closed polylines, vertex verification etc. before allowing the scrutiny engine to extract the drawing data from submitted drawings.

The Pre-Check engine does not scrutinize the drawing with respect to any regulations. This engine only ensures that the file is geometrically correct and ready to be scrutinized in the next step.

When the Architect / Consultants submit the building plan proposal, this function enables the architect to identify the defects in the drawing and make it suitable to run through the Scrutiny Engine. It reads the drawing elements and lists all the discrepancies in the drawing. If errors are found in the drawing, the drawing will be populated with hyperlinks.

Following are some of the types of errors displayed in the Pre-Check returned drawing file.

    - Multiple Polyline Geometry Problems

    - Invalid object color.

    - Existence of Blocks, Regions, Wipe-outs & External references in Floor layers.

    - Mandatory requirements

    - Other critical errors

By clicking on the above disqualified items in the software interface, details will be displayed such as disqualified object name; floor name and drawing coordinate position. It will also draw a crossed grey line on all the disqualified objects throughout the drawing.

  - SCRUTINY ENGINE

Once the pre-check is passed, the file automatically moves to the scrutiny engine. The Scrutiny Engine compares the rules of SIDA and Amendments with the extracted data from the Submitted drawing and generates a Scrutiny Report listing out each rules of SIDA and Amendments parameter.

- PROCEDURE TO CREATE A DRAWING

Once the drawing is ready, architect / Consultants has to add layers, colors, text based on his Building type requirement.

Refer to appendix for screenshots of sample color coding layers. Please note that these are only to view the color coding, and not to be used as a reference for any of the architectural elements.

- Put the entire drawing in 0 layer

- Make new layers based on the floors and color coding based on the building type requirements, .

- Compulsory labels – Must label all floors, must label all blocks, must provide building use.

  - DO’S & DON’T’S

The processing of the file and further generation of reports is dependent on the Polylines, Lines and Text placed in the drawing file.

    - DO’S

- Bounding rectangle should be kept as a polyline containing the whole submission drawing with floor plans, site detail, key plan, foundation detail, culvert detail, rainwater harvesting detail, compound wall detail etc. The lower left corner of the bounding rectangle to be kept in 0,0,0 Co-ordinate. Any drawings / objects placed outside this box will not be scrutinized.

- Drawing objects shall be placed in model space only.

- After completing color coding, check the drawing using Laywalk command to identify Whether the color coded objects are in respective layers.

- Before File submission in online, Unlock, Unfreeze and turn On, Purge all unwanted Layers.

- All the screen shots in the manual are for representation purpose only, It is the Prime Responsibility of the Architect / Consultants to ensure that the Building Plan is prepared in Compliance to Rules.

      - LAYERING SYSTEM

- Proposed Floor layer: FLOOR01, FLOOR02, etc…. corresponding to the floor count.

- FLOOR-PARKING/ STILT FLOOR or FLOOR-GROUND for common Site related details.

- For Proposed Basement floor, layer should be FLOOR-BF1, FLOOR–BF2 etc. corresponding to the Basement Floor Count.

      - DRAWING FILE SPECIFICATIONS

Proposal Drawing should be in (.DWG) Format.

        - All objects to be processed by the system should be drawn using Lines, Polylines and Text and placed in layers starting with Floor.

        - Layers and Colors should be followed as per the color code table.

        - Purge the unused layers, dimension styles etc.

        - Drawing units should be in meters.

        - Avoid unnecessary objects in the drawing.

        - Avoid unnecessary coordinates in polylines.

        - Don’t Overlap the Objects.

    - DON’T’S

- Outside the bounding rectangle, objects should not be kept. OLE(Object Linking and embedding) objects should not be used.

- All drawing objects shall be in 2 Dimensional (Z-Coordinates – Zero) and placed without any elevation in x y plane (Top view) do not import the data from 3d CAD Software.

- Blocks, Regions, Wipeout and External references shall not be placed in the drawing as they will not be processed.

- The Color-coded items, i.e. polylines, lines and texts which are intended to be processed must be in the layers starting with FLOOR. Other layers will be ignored by the software

- Don’t upload password protected cad drawing

- DETAILS OF COLOR CODE FORMAT

| OBJECTS TO BE DRAWN IN SITE PLAN OR FLOOR PLAN LAYER NAME TO BE FOLLOWED - FLOOR-GROUND (OR) FLOOR-STILT (OR) FLOOR- UPPER GROUND (OR) FLOOR- UNDER GROUND (OR) FLOOR-BF (OR) FLOOR-PARKING (OR) FLOOR- PODIUM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S. No. | Description | Color Code |  | Object Type | Layer |
| 1 | Plot Boundary | 7 | White | Polyline | FLOOR-GROUND (OR) FLOOR-STILT |
| 2 | Setback Boundary/Building Area/Building Line | 10 |  | Polyline | FLOOR-GROUND (OR) FLOOR-STILT |
| 3 | Setback (Front) | 4 |  | Line | FLOOR-GROUND (OR) FLOOR-STILT |
|  | Setback (Rear) | 3 |  |  |  |
|  | Setback (Side1) | 6 |  |  |  |
|  | Setback (Side2) | 2 |  |  |  |
| 4 | Plot Frontage / Plot Width | 96 |  | Open Polyline | Floor-Ground (Or) Floor-Stilt |
| 5 | Road Width | 41 |  | Line | FLOOR-GROUND (OR) FLOOR-STILT |
| 6 | Soft Landscape or Open Area | 60 |  | Polyline | FLOOR-GROUND (OR) FLOOR-STILT |
| 7 | Trees | 93 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 8 | Plantation | 57 |  | Polyline | FLOOR-GROUND (OR) FLOOR-STILT |
| 9 | Street Parking for School | 183 |  | Polyline | FLOOR-GROUND (OR) FLOOR-STILT |
| 10 | Main Gate | 161 |  | Line | FLOOR-GROUND (OR) FLOOR-STILT |
| 11 | Guard Room | 182 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 12 | Servant Quarters | 181 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 13 | Canopy | 184 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 14 | Watchman Shelter | 185 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 15 | Watch Tower | 186 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 16 | Pump Room | 187 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 17 | Air conditioning room | 188 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 18 | Store | 189 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 19 | Garage | 190 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 20 | Open Transformer | 191 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 21 | Meter Room | 192 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 22 | Under Ground Water Tank | 193 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 23 | Riser | 27 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 24 | Tread | 107 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 25 | Flight/Landing | 29 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 26 | Open Parking Area | 20 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 27 | Height Of the Building after exemptions | 151 |  | Line | FLOOR-GROUND (OR) FLOOR-STILT |
| 28 | Floor to Floor Height | 233 |  | Line | FLOOR-GROUND (OR) FLOOR-STILT |
| 29 | Plinth Height | 105 |  | Line | FLOOR-GROUND (OR) FLOOR-STILT |
| 30 | Boundary Wall Height | 160 |  | Line | FLOOR-GROUND (OR) FLOOR-STILT |
| 31 | Block Distinguishers | 50 |  | Polyline & Text | FLOOR-GROUND (OR) FLOOR-STILT |
| 32 | Loading and Unloading area | 221 |  | Polyline | FLOOR-GROUND (OR) FLOOR-STILT |
| 33 | Stilt Floor Boundary | 5 |  | Polyline | FLOOR-STILT |
| 34 | Stilt Floor Height at the Bottom of the beam | 21 |  | Line | FLOOR-STILT |
| 35 | Stilt Floor Height at the bottom of slab | 91 |  | Line | FLOOR-STILT |
| 36 | Basement Boundary | 32 |  | Polyline | FLOOR-BF1 (OR) FLOOR- BF2………. |
| 37 | Basement Floor Height at the Bottom of the Beam | 23 |  | Line | FLOOR-BF1 (OR) FLOOR- BF2………. |
| 38 | Basement Floor Height | 90 |  | Line | FLOOR-BF1 (OR) FLOOR- BF2………. |
| 39 | Car Parking | 140 |  | Polyline | APPLICABLE FLOOR LAYER |
| 40 | Scooter Parking | 101 |  | Polyline | APPLICABLE FLOOR LAYER |
| 41 | Ground Floor area | 215 |  | Polyline | APPLICABLE FLOOR LAYER |
| 42 |  | Vehicular Ramp | 106 |  | Polyline & Text | APPLICABLE FLOOR LAYER |
| 43 |  | Lift | 22 |  | Polyline | APPLICABLE FLOOR LAYER |
| 44 |  | Stair | 115 |  | Polyline & Text | APPLICABLE FLOOR LAYER |
| 45 |  | Fire Stair | 116 |  | Polyline & Text | APPLICABLE FLOOR LAYER |
| 46 |  | Duct / shaft | 11 |  | Polyline & Text | APPLICABLE FLOOR LAYER |
| 47 |  | Double height | 12 |  | Polyline & Text | APPLICABLE FLOOR LAYER |
| 48 |  | Pergola | 63 |  | Polyline & Text | APPLICABLE FLOOR LAYER |
| 49 |  | Fire Corridor | 243 |  | Polyline & Text | APPLICABLE FLOOR LAYER |
| 50 |  | Mechanized Parking | 213 |  | Polyline & Text | APPLICABLE FLOOR LAYER |
| 51 |  | Loft | 120 |  | Polyline & Text | APPLICABLE FLOOR LAYER |
| 52 |  | Ventilation | 128 |  | Polyline | APPLICABLE FLOOR LAYER |
| 53 |  | Cantilever Balcony | 35 |  | Polyline | APPLICABLE FLOOR LAYER |
| 54 |  | Roof Top Water Tank | 162 |  | Polyline | APPLICABLE FLOOR LAYER |
| 55 |  | Chajja Projection | 85 |  | Polyline | APPLICABLE FLOOR LAYER |
| 56 |  | Cantilever Projection | 219 |  | Polyline | APPLICABLE FLOOR LAYER |
| 57 |  | Decorative Column Projection | 113 |  | Polyline | APPLICABLE FLOOR LAYER |
| 58 |  | Moulding Cornice | 71 |  | Polyline | APPLICABLE FLOOR LAYER |
| 59 |  | Parapet wall | 99 |  | Polyline | APPLICABLE FLOOR LAYER |
| 60 |  | RWH Height | 92 |  | Line | APPLICABLE FLOOR LAYER |
| 61 |  | Mechanized Parking Level Height | 236 |  | Line | APPLICABLE FLOOR LAYER |
| 62 |  | Loft Height | 136 |  | Line | APPLICABLE FLOOR LAYER |
| 63 |  | Typical Floor Text – Type 1 | 7 | White | Text | APPLICABLE FLOOR LAYER |
| 64 |  | Typical Floor Text – Type 2 | 7 | White | Text | APPLICABLE FLOOR LAYER |
| 65 |  | Mezzanine Floor | 180 |  | Polyline | FLOOR-MFGROUNDTO01 |
| 66 |  | Mezzanine Floor Height | 88 |  | Line | FLOOR-MFGROUNDTO01 |
| 67 |  | Service Floor | 8 |  | Polyline & Text | FLOOR-SERVICE |
| 68 |  | Service Floor Height | 89 |  | Line | FLOOR-SERVICE |
| 69 |  | Mumty | 9 |  | Polyline & Text | FLOOR-TERRACE |
| 70 |  | Chimney | 13 |  | Polyline & Text | FLOOR-TERRACE |
| 71 |  | Lift Machine Room | 14 |  | Polyline & Text | FLOOR-TERRACE |
| 72 |  | Stilt Parking | 15 |  | Polyline & Text |  |
| 73 |  | Basement Parking | 31 |  | Polyline & Text |  |
| 74 |  | RWH Tank | 94 |  | Polyline & Text |  |
| 75 |  | Mumty Height | 45 |  | Line |  |
| 76 |  | Machine Room Height | 46 |  | Line |  |
| 77 |  | Typical Floor area | 194 |  | Polyline & Text |  |
| 78 |  | First Floor Area | 195 |  | Polyline & Text |  |
| 79 |  | Second Floor Area | 196 |  | Polyline & Text |  |
| 80 |  | Third Floor Area | 197 |  | Polyline & Text |  |
| 81 |  | Fourth Floor Area | 198 |  | Polyline & Text |  |
| 82 |  | Fifth Floor Area | 199 |  | Polyline & Text |  |
| 83 |  | Sixth Floor Area | 200 |  | Polyline & Text |  |
| 84 |  | Seventh Floor Area | 201 |  | Polyline & Text |  |
| 85 |  | Eighth Floor Area | 202 |  | Polyline & Text |  |
| 86 |  | Ninth Floor Area | 203 |  | Polyline & Text |  |
| 87 |  | Tenth Floor Area | 204 |  | Polyline & Text |  |
| 89 |  | Eleventh Floor Area | 205 |  | Polyline & Text |  |
| 90 |  | Twelfth Floor Area | 206 |  | Polyline & Text |  |
| 91 |  | Thirteenth Floor Area | 207 |  | Polyline & Text |  |
| 92 |  | Fourteenth Floor Area | 208 |  | Polyline & Text |  |
| 93 |  | Fifteenth Floor Area | 209 |  | Polyline & Text |  |
| 94 |  | Sixteenth Floor Area | 210 |  | Polyline & Text |  |
| 95 |  | Seventeenth Floor Area | 211 |  | Polyline & Text |  |
| 96 |  | Eighteenth Floor Area | 212 |  | Polyline & Text |  |
| 97 |  | Ground level | 18 |  | Line |  |
| 98 |  |  |  |  |  |  |